TEMT · TRANSPORTATION EMISSION MEASUREMENT TOOL

# Freight emissions report

Sample FMCG company (synthetic data)

FY 2025–26 · Generated 27 September 2026 · GLEC Framework v3.2 · India defaults · engine temt-engine-2.0.0

# Summary

| Measure | Value |
| --- | --- |
| Total emissions, well-to-wheel | 325.5 t CO₂e |
| Tank-to-wheel (vehicle operation) | 241.4 t CO₂e |
| Well-to-tank (energy provision) | 78.95 t CO₂e |
| Hubs and terminals | 5.21 t CO₂e |
| Shipments and legs | 169 shipments · 334 legs |
| Emission intensity | 22.4 g CO₂e per tonne-km |
| Emissions per tonne shipped | 66.6 kg CO₂e |
| Change vs FY 2024–25 | -8.4% |

# Key insights

- Road is your largest source. 68% of emissions from 24% of tonne-kilometres.

- Your heaviest lane. Hosur → Delhi accounts for 12% of emissions across 6 shipments.

# Emissions by transport mode

| Mode | t CO₂e | Share | Tonne-km |
| --- | --- | --- | --- |
| Road | 222.9 | 68.5% | 35,24,570 |
| Rail | 22.8 | 7% | 21,71,296 |
| Air | 6.43 | 2% | 5,769 |
| Sea | 68.15 | 20.9% | 88,57,481 |
| Hubs and terminals | 5.21 | 1.6% | 0 |

# GHG Protocol scopes

| Scope | t CO₂e | Share | Tonne-km |
| --- | --- | --- | --- |
| Scope 3 · Category 4 | 280.8 | 86.2% | 1,07,78,433 |
| Scope 3 · Category 9 | 42.56 | 13.1% | 37,55,646 |
| Scope 1 · own fleet fuel | 2.17 | 0.7% | 24,575 |
| Scope 2 · own fleet electricity | 0.04 | 0% | 463 |

# Business units

| Business unit | t CO₂e | Share | Tonne-km |
| --- | --- | --- | --- |
| Personal Care | 112.1 | 34.4% | 49,65,753 |
| Home Care | 109.4 | 33.6% | 53,10,548 |
| Foods | 104 | 31.9% | 42,82,815 |

# Top lanes

| Lane | t CO₂e | Share | Tonne-km |
| --- | --- | --- | --- |
| Hosur → Delhi | 37.62 | 11.6% | 16,01,320 |
| Hosur → Jaipur | 20.54 | 6.3% | 7,70,691 |
| Hosur → Rotterdam | 20.11 | 6.2% | 17,60,073 |
| Hosur → Kochi | 19.83 | 6.1% | 7,30,877 |
| Baddi → Chennai | 18.22 | 5.6% | 4,71,662 |
| Baddi → Hyderabad | 15.28 | 4.7% | 2,94,057 |
| Baddi → Rotterdam | 13.95 | 4.3% | 10,80,446 |
| Silvassa → Delhi | 13.75 | 4.2% | 10,19,633 |
| Baddi → Kolkata | 12.45 | 3.8% | 2,62,205 |
| Hosur → Chennai | 11.07 | 3.4% | 3,53,541 |

# Data quality

| Basis | t CO₂e | Share | Tonne-km |
| --- | --- | --- | --- |
| Default factors | 323.3 | 99.3% | 1,45,34,079 |
| Primary data (measured fuel, energy or carrier value) | 2.21 | 0.7% | 25,038 |

# Reduction opportunities

- Move long road hauls to rail (up to 148.7 t CO₂e). 53 road legs of 500 km or more with at least 8 t of cargo could move by rail with 30 km road drayage at each end. Biggest lanes: Hosur → Delhi; Baddi → Chennai; Hosur → Jaipur. Check rail access, capacity and transit time before committing.

- Replace short air legs with road (up to 5.44 t CO₂e). 2 air legs of up to 2,500 km. Moving time-flexible consignments to an intermediate truck would avoid most of their emissions.

- Consolidate long hauls into larger trucks (up to 0.65 t CO₂e). 6 long-haul legs use trucks of 12 t GVW or less. Consolidating loads into a 12–20 t truck lowers emissions per tonne-kilometre.

- Replace default factors with primary data. 98% of emissions use default factors. Ask your top carriers for fuel or energy records; ISO 14083 prefers primary data and it usually shows the effect of your own improvements.

# BRSR mapping

| Indicator | Value | Note |
| --- | --- | --- |
| Principle 6 · Scope 3 GHG emissions (freight share) | 323.31 t CO₂e | Leadership indicator. Category 4 and Category 9 transport, well-to-wheel. |
| Principle 6 · Scope 1 and 2 GHG emissions (own fleet share) | 2.21 t CO₂e | Own fleet fuel (Scope 1) and electricity for own electric vehicles (Scope 2). Add to your site-level Scope 1 and 2 inventory. |
| Freight emission intensity | 22.4 g CO₂e per tonne-km | Physical intensity for year-on-year comparison. |
| Freight emissions per ₹ crore of revenue | 0.026 t CO₂e | Revenue entered in Settings: ₹12,500 crore. |
| Methodology | GLEC v3.2 India | ISO 14083-aligned; factor sources listed in the methodology section. |

# Methodology

GLEC Framework v3.2 · India defaults

Latest published defaults (October 2025). Indian road intensities originate from TCI–IIMB Supply Chain Sustainability Lab research; rail, air, sea, inland waterway and hub values follow GLEC Module 2.

- Emissions are reported well-to-wheel (WTW) in kg or tonnes CO₂e, split into tank-to-wheel (vehicle operation), well-to-tank (energy provision) and hub operations, following ISO 14083:2023 and the GLEC Framework v3.2.

- Default factors are estimates for typical operations. Replace them with primary fuel or energy data from carriers where available.

- Distances marked as estimated come from straight-line distance multiplied by network factors or from sea-lane routing; actual distances were used where supplied.

- Scope classification follows the GHG Protocol: own fleet → Scope 1/2; purchased transport → Scope 3 Category 4; customer-paid downstream transport → Scope 3 Category 9.

- Production TEMT's ISO 14083 and ISO/IEC 27001:2022 certifications apply to that platform and its stated scope.

## Factors used

| Factor | WTT | TTW | Unit | Source |
| --- | --- | --- | --- | --- |
| Heavy rigid truck, 20–30 t GVW, diesel | 0.0191 | 0.0632 | kg CO₂e/t-km | Smart Freight Centre, Table 13 |
| Multi-axle heavy truck, 30–50 t GVW, diesel | 0.0154 | 0.0509 | kg CO₂e/t-km | Smart Freight Centre, Table 13 |
| Indian Railways average, mixed diesel/electric traction | 0.0064 | 0.0041 | kg CO₂e/t-km | Smart Freight Centre, Module 2, rail, Region: India (mixed diesel/electric traction) |
| Tractor-trailer, 30–60 t GVW, diesel | 0.0128 | 0.0423 | kg CO₂e/t-km | Smart Freight Centre, Table 13 |
| Light commercial vehicle, up to 3.5 t GVW, diesel | 0.0879 | 0.2916 | kg CO₂e/t-km | Smart Freight Centre, Table 13 |
| Medium rigid truck, 12–20 t GVW, diesel | 0.0209 | 0.0692 | kg CO₂e/t-km | Smart Freight Centre, Table 13 |
| Container, Intra Middle East / India (coastal and regional), dry | 0.00141 | 0.00648 | kg CO₂e/t-km | Smart Freight Centre, Sea Table 18, container end-user values |
| Intermediate rigid truck, 5–12 t GVW, diesel | 0.0324 | 0.1075 | kg CO₂e/t-km | Smart Freight Centre, Table 13 |
| Small commercial vehicle, 3–5 t GVW, diesel | 0.0514 | 0.1707 | kg CO₂e/t-km | Smart Freight Centre, Table 13 |
| Diesel (HSD) | 0.97 | 3.24 | kg CO₂e/kg | Smart Freight Centre, Module 1, Indian sources |
| Container, Europe (North & Med) ↔ Middle East / India, dry | 0.0013 | 0.00604 | kg CO₂e/t-km | Smart Freight Centre, Sea Table 18, container end-user values |
| Container, Industry average (lane unknown), dry | 0.00127 | 0.0059 | kg CO₂e/t-km | Smart Freight Centre, Sea Table 18, container end-user values |
| Intermediate rigid truck, 5–12 t GVW, CNG | 0.0459 | 0.1229 | kg CO₂e/t-km | Smart Freight Centre, Table 13 |
| Grid electricity, India | 0.71 | 0 | kg CO₂e/kWh | Central Electricity Authority, Ministry of Power, Weighted average, FY 2024–25 |
| Container, Intra Middle East / India (coastal and regional), dry | 0.00205 | 0.0094 | kg CO₂e/t-km | Smart Freight Centre, Sea Table 18, container end-user values |
| Container, Asia ↔ Middle East / India, dry | 0.00138 | 0.00645 | kg CO₂e/t-km | Smart Freight Centre, Sea Table 18, container end-user values |
| Air freight, belly hold, long-haul | 0.161 | 0.775 | kg CO₂e/t-km | Smart Freight Centre, Air Table 1 |
| Air freight, belly hold, short-haul | 0.213 | 1.026 | kg CO₂e/t-km | Smart Freight Centre, Air Table 1 |

## Sources

- Smart Freight Centre (2025). Global Logistics Emissions Council Framework v3.2. https://smartfreightcentre.org/news/13311209

- Central Electricity Authority, Ministry of Power (2025). CO2 Baseline Database for the Indian Power Sector, Version 21.0. https://cea.nic.in/cdm-co2-baseline-database/?lang=en

- International Organization for Standardization (2023). ISO 14083:2023 Greenhouse gases — Quantification and reporting of GHG emissions arising from transport chain operations. https://www.iso.org/standard/78864.html

- World Resources Institute and WBCSD (2011). Corporate Value Chain (Scope 3) Accounting and Reporting Standard. https://ghgprotocol.org/corporate-value-chain-scope-3-standard